Workplace inspection

Canada — field sheet. Ontario sets a monthly interval in statute; British Columbia sets a standard instead.

What this checklist is not It is not a prescribed form — no Canadian jurisdiction prescribes one. Most of the lines below are trade practice and say so; what is required is the thing each line checks, not the checking. But the *inspection itself* is a legal duty in several provinces, with an interval attached, which is the subject of section 2. Nothing here is legal advice.

1. Scope of this inspection

— industry practice, nothing requires it

Fill the scope in before you set off. An inspection whose area nobody can identify six months later has stopped being evidence of anything — and that is the commonest defect of inspections that were done properly.

| Site |  | Area or department inspected |  |
| --- | --- | --- | --- |
| Date |  | Start and finish time |  |
| Type | [monthly · special (malfunction or accident) · topic · new installation] | Workers present in the area |  |
| Participants | [names and roles — see section 2] |  |  |

2. What the law actually requires

— required by law

Ontario is the outlier and puts a number in the statute: a designated worker member of the joint committee — or the health and safety representative in a smaller workplace — «shall inspect the physical condition of the workplace at least once a month» (OHSA ss. 9(26) and 8(6)). British Columbia sets a standard instead of an interval: regular inspections of all workplaces «at intervals that will prevent the development of unsafe working conditions» (OHS Regulation s. 3.5), plus a special inspection when required by malfunction or accident.

Who should be present

In Ontario the inspecting person is designated by the worker members of the committee, and in British Columbia the committee or representative takes part where feasible. A walk done by one person from the safety office finds less and gets less fixed.

- A designated worker member of the joint committee, or the health and safety representative

- An employer representative

- The supervisor of the area walked

- A worker who does the job, where the walk covers a workstation

- Where neither a committee nor a representative exists in BC, one employer-designated and one worker-designated representative (OHS Regulation s. 3.8)

3. Field checklist

— industry practice, nothing requires it

Tick what is in order, leave blank what is not, and carry every blank into the findings table. An ambiguous tick is a finding.

Delete the themes that do not apply to the area before printing. A forty-line sheet with fifteen irrelevant lines gets filled in badly, and the assembler on the template page exists for exactly that.

Access, floors and traffic

|  | Traffic routes clear and marked. |
| --- | --- |
|  | Floors free of holes, standing liquid and trailing cables. |
|  | Pedestrian and mobile-equipment routes actually separated on the ground. |
|  | Lighting adequate, including in seldom-used passageways. |

Exits, first aid and fire

|  | Exits and egress routes unobstructed and unlocked in the direction of travel. |
| --- | --- |
|  | Emergency procedures posted and current. |
|  | Fire extinguishers in place, accessible, inspection current. |
|  | First aid attendant on shift and first aid supplies at the required level. The required attendant level and equipment depend on the province, the number of workers and the distance to care — check the schedule for your jurisdiction rather than assuming a kit is enough. |

Machines and equipment

|  | Guards in place and not defeated. A bypassed guard is a same-day finding, not a next-walk item. |
| --- | --- |
|  | Emergency stops reachable and identified. |
|  | Lockout devices and written procedures available at the machine. |
|  | Pre-use inspection records for mobile equipment present and current. |

Hazardous products (WHMIS)

|  | Supplier labels intact; workplace labels on every decanted container. WHMIS. A decanted container with no label is the classic finding, and the fix is a marker pen. |
| --- | --- |
|  | Safety data sheets accessible to workers in the area. |
|  | Storage compatible, spill containment in place, ventilation working. |
|  | No unidentified product in the area. |

Working at height and handling

|  | Guardrails, floor openings and covers protected. |
| --- | --- |
|  | Ladders used for access, not as a work platform. |
|  | Anchor points identified and inspected where fall arrest is used. |
|  | Handling aids available and actually used. |

PPE

|  | PPE worn, correct for the hazard, and in serviceable condition. |
| --- | --- |
|  | PPE available in the right size at the point of use. |
|  | Respirator fit tests current for everyone using one. |

Posting and records

|  | A copy of the Act and the explanatory material posted in a conspicuous place, or in a readily accessible electronic format. Ontario OHSA s. 25(2)(i). The electronic option is in the statute — a link people can actually reach counts. |
| --- | --- |
|  | The written health and safety policy posted. Ontario OHSA s. 25(2)(k), above five workers. |
|  | Committee membership and minutes posted where required. |
|  | Safe work procedures present and legible at the workstation. |

4. Findings and actions

— industry practice, nothing requires it

A finding with no owner and no date has never been fixed. That is the only thing this table exists to prevent.

| No. | Finding | Theme | Severity | Action decided | Owner | Due |
| --- | --- | --- | --- | --- | --- | --- |

Anything exposing somebody right now — a defeated guard, a blocked exit, an unidentified product — is dealt with before the walk ends, not entered into a ninety-day plan.

5. Follow-up

— required by law

Ontario closes the loop in the statute: the inspecting member «shall inform the committee of situations that may be a source of danger or hazard to workers and the committee shall consider such information within a reasonable period of time» (OHSA s. 9(30)). The committee's duty to *consider* is the part most often skipped.

|  | Findings recorded and given to the supervisor of the area. |
| --- | --- |
|  | Committee informed of situations that may be a source of danger, and the committee has considered them. OHSA s. 9(30) — and the consideration is a duty on the committee, within a reasonable period. |
|  | Inspection records kept as part of the health and safety programme. |
|  | Items requiring immediate correction dealt with before the walk ended. |
|  | Next inspection date set and recorded against the schedule. |

6. Sign-off

— industry practice, nothing requires it

A signature does not certify that the area is compliant. It records that the inspection happened, with these people, and that these were the findings.

| Worker member or representative Name, date | Employer representative Name, title, date | Supervisor of the area Name, date |
| --- | --- | --- |

Sources

- Ontario OHSA, ss. 8 and 9 — inspections by the representative and the committee — https://www.ontario.ca/laws/statute/90o01

- BC OHS Regulation, Part 3 — ss. 3.5, 3.7 and 3.8 — https://www.worksafebc.com/en/law-policy/occupational-health-safety/searchable-ohs-regulation/ohs-regulation/part-03-rights-and-responsibilities

Content last reviewed 2026-09-08 · This template's page — https://safeally.net/en-ca/templates/workplace-inspection-checklist/

SafeAlly — safeally.net · free, no account, editable document